METKLEAN PRODUCTS			SAFETY DATA SHEET

1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND COMPANY

### PRODUCT NAME	:	POWERSAN 7

### SUPPLIED BY	:	METKLEAN PRODUCTS LIMITED

## 23 HAILEY ROAD

HAILEY ROAD BUSINESS PARK

ERITH    KENT    DA18 4AA

# Tele: 020 8310 9882/8642

Fax:  020 8312 1336

2. GENERAL COMPOSITION

A BLEND OF CATIONIC QUATERINARY SURFACTANTS <2%, NON-IONIC SURFACTANT 2 - 5%,

PHOSPHATES, SILICATES, SEQUESTRANTS IN DYED AQUEOUS SOLUTION.

3. HAZARD IDENTIFICATION

IRRITATING TO MUCOUS MEMBRAMES.

4. FIRST AID MEASURES

Inhalation		:	NOT CONSIDERED A PROBLEM.  IF AFFECTED REMOVE PATIENT TO FRESH

AIR.

Skin Contact		:	WASH OFF WITH CLEAN WATER.

Eye Contact		:	IMMEDIATELY WASH OUT WITH PLENTY OF WATER FOR AT LEAST

10 MINUTES.  SEEK MEDICAL ATTENTION IF IRRITATION PERSISTS.

Ingestion		:	WASH OUT MOUTH WITH WATER.  GIVE PLENTY OF WATER TO DRINK.

OBTAIN MEDICAL ADVICE.

METKLEAN PRODUCTS				SAFETY DATA SHEET

5. FIRE FIGHTING MEASURES

Extinguishing Media			:	SELECT EXTINGUISHING MEDIUM APPROPRIATE TO OTHER

MATERIALS INVOLVED.

Hazardous Combustion Products	:	UNDER EXTREME CONDITIONS OF HEAT, DECOMPOSITION

COULD GENERATE ACRID AND TOXIC FUMES.  SELF

CONTAINED BREATHING APPARATUS SHOULD BE WORN.

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions		:	WEAR IMPERVIOUS GLOVES/OVERALLS AND BOOTS WHILE

CLEARING UP THE SPILLAGE.

Environmental Precautions	:	AVOID SURFACE DRAINS AND WATER COURSES.  IF SPILLAGES

OCCUR IT MAY BE NECESSARY TO ADVISE THE APPROPRIATE

AUTHORITIES.

Spillage			:	FLUSH TO DRAIN WITH PLENTY OF WATER.

7. HANDLING AND STORAGE

Handling	:	AVOID CONTACT WITH CONCENTRATE.

Storage	:	AMBIENT CONDITIONS.

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Exposure Limits	:	NONE ESTABLISHED.

#### Personal Protective Equipment

Respiratory		:	NOT REQUIRED.

Hand 			:	PVC OR RUBBER GLOVES RECOMMENDED.

Eye protection		:	SAFETY GOGGLES RECOMMENDED IF SPLASHES ARE LIKELY TO OCCUR.

Skin protection	:	PROTECTIVE OVERALLS.

Other			:	NONE REQUIRED.

METKLEAN PRODUCTS						SAFETY DATA SHEET

9. PHYSICAL AND CHEMICAL PROPERTIES

Appearance			:	CLEAR FLUORESCENT GREEN MOBILE LIQUID

Odour				:	FAINT

pH				:	12.0

Specific Gravity		:	1.09

##### Flash Point			:	NON-FLAMMABLE

Volatility			:	NON-VOLATILE

##### Hazardous Components	:	NONE

##### Classification			:	NON CLASSIFIED

##### Solubility in Water		:	YES, IN ALL PROPORTIONS

(THIS IS NOT A SPECIFICATION)

10. STABILITY AND REACTIVITY

Stability			:	STABLE

Conditions to Avoid		:	EXCESSIVE CONDITIONS OF COLD AND HEAT

11. TOXICOLOGICAL INFORMATION

EYE CONTACT		:	WOULD CAUSE SKIN IRRITATION.  WASH WELL WITH FRESH WATER FOR

10 MINUTES AND SEEK MEDICAL ADVICE IF DISCOMFORT PERSISTS.

SKIN CONTACT	:	PROLONGED CONTACT WOULD CAUSE SKIN IRRITATION, SENSITISATION

AND DE-FATTING.  WASH SPLASHES WITH CLEAN WATER.

INHALATION		:	NOT APPLICABLE.  VERY LOW ODOUR.

#### INGESTION 		:	WOULD RESULT IN IRRITATION OF THE MOUTH, THROAT AND DIGESTIVE

##### TRACT.  DRINK PLENTY OF WATER OR MILK AND SEEK MEDICAL ADVICE.

METKLEAN PRODUCTS						SAFETY DATA SHEET

12.                                                ECOLOGICAL INFORMATION

Degradability		:	FULLY BIODEGRADABLE.

Bio Accumulation	:	NONE.

Aquatic Toxicity	:	NO DATA.  DISCHARGE OF LARGE QUANTITIES INTO THE AQUATIC

ENVIRONMENT MAY KILL FISH AND OTHER AQUATIC ORGANISMS.

13. DISPOSAL CONSIDERATIONS

Disposal Methods		:	VIA AN AUTHORISED WASTE DISPOSAL CONTRACTOR IN

ACCORDANCE WITH LOCAL AND NATIONAL REGULATIONS.

Contaminated Packaging	:	THOROUGHLY RINSED CONTAINERS MAY BE DISPOSED OF AS

GENERAL RUBBISH.

14. TRANSPORT INFORMATION

###### Packaging 				:	4 x 5 LITRE BOXES

###### (Size and Description)				25 LITRE POLY KEGS

Transport Classification		:	NON HAZARDOUS

###### Substance Identification Number	:	NOT CLASSIFEID

Packing group				:	NOT APPLICABLE

Transport Hazard Symbol		:	NONE

METKLEAN PRODUCTS						SAFETY DATA SHEET

15. REGULATORY INFORMATION

###### Supply classification			:	NON CLASSIFIED

###### Hazard classification			:	NON CLASSIFIED

###### Hazard Pictogram			:	NOT APPLICABLE

###### Risk phrases				:	NONE

Safety phrases				:	NONE

Other Applicable Regulations		:	NONE RECOMMENDED

16. OTHER INFORMATION

The information contained in this document, only covers the hazards presented by this material, it does not constitute a Workplace Risk Assessment.

Disclaimer

This Safety Data Sheet is based on current knowledge and relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process. Such information is, to the best of our knowledge and belief, accurate and reliable as of the date indicated. However, no representation, warranty or guarantee is made as to its accuracy, reliability, or completeness. It is the user's responsibility to satisfy himself as to the suitability of such information for his own particular use.

Revision No		Sept 09

Replaces SDS

Reference _	: CHIPS\POWERSAN 7